Table 1: The Spaulding Classification System for decontamination.

| Risk level | Process | Storage |
| --- | --- | --- |
| Critical (high risk) Definition of a critical medical device: Item intended to be introduced directly into or have contact with the vascular system or normally sterile areas in the body. Examples may include: Instruments used in minor surgery and probes. | Clean As soon as possible after using Sterilise by moist heat after cleaning If an RMD is heat or moisture sensitive, sterilise using an alternative low temperature sterilisation process. | - Sterility to be maintained - Packaged RMDs are to be stored to prevent environmental contamination in a designated storage area - RMDs processed through a liquid chemical sterilisation process are to be used immediately |
| Semi critical (medium risk) Definition of semi critical medical device: a device that comes into contact with mucous membranes or non-intact skin. Examples may include instruments for wound care and debridement such as tweezers and scissors. | Clean as soon as possible after using. Sterilised by moist heat after cleaning If an RMD is unable to withstand the process variables of moist heat sterilisation, sterilise with a low temperature sterilisation process, or high level disinfection process. | - Store to prevent environmental contamination in a designated storage system, for example, a controlled environment drying cabinet. |
| Non critical (low risk) Definition of a non critical medical device: A medical device that comes into contact with intact skin but not mucous membranes. Examples may include a blood pressure cuffs, stethoscopes, reflex hammers, tuning forks, scales etc | Clean as soon as possible after using For further treatment, just disinfect with compatible low level or intermediate level instrument grade disinfectant or thermal disinfection. | - RMDs to be stored in a clean dry place to minimise environmental contamination |